SPECIFICATION

FOR

MANUAL METAL ARC WELDING MACHINE

      - TYPE:-Thyristorised control portable rectifier suitable for manual arc welding     process.

2. SPECIFICATION:-

- Input voltage:      415 +  10%  variation

Phase           :         3 phase

Frequency    :     50Hz     +   3% variation

- Welding current range                   :    20-600 Amp

Output current at 100% duty cycle:    460 Amp

Output current at 60% duty cycle  :    600 Amp

- Open circuit  voltage at set current:    100  V

- Output current variation at set current: +   1% (max.)

- Class of insulation :  class “H”

- Cooling  :  Forced Air

- Type of control  : Step less current variation

  - TECHNICAL FEATURES:-  The rectifier should have following in built features:

    - Constant current characteristics.

    - Easiness in starting and restarting of arc.

    - Excellent arc stability throughout the entire range (20-600 Amp).

    - Effective fan cooling to ensure complete cooling of all power components.

    - Under voltage & over voltage protection to take care for input voltage fluctuation.

    - Protection against single phasing, short circuit, thermal overload and fan failure.

    - Hot start / anti stick characteristics to prevent electrode freezing.

    - Easy accessibility to major components from maintenance viewpoint.

    - Compact & robust in construction for use at rough shop floor condition.

    - Fitted with Ammeter & voltmeter.

    - Step less current variation through knob on front panel of rectifier and also through remote control.

  - SCOPE OF SUPPLY:-

- Individual price should be mentioned against each item.

- Welding rectifier as per specification.

- Remote control unit with 20.0 m cable & connector.

- Input cable 5.0 m long with plug/connector.

- Earthing cable 5.0 m long with earthing clamp.

- Toolbox consisting of spanners, allen key, pliers etc.

- Recommended spares for 2 years smooth & trouble free working indicating individual identification no & price including following items:-

  - Thyristor /power diodes.

  - Control PCB.

    - OPERATION AND MAINTENANCE MANUALS:-

Five sets of operation & maintenance manuals consisting of following:-

- Operating & maintenance instruction including do’s & don’ts.

- Mechanical assembly diagrams.

- Electrical & electronics circuit diagrams and writing diagrams.

- PCB circuit diagrams.

- Fault diagnostics & remedies.

- List of complete spares with identification nos.

  - Commissioning & trial testing at BHEL works.

  - Guarantee for 3 years smooth working.

  - Party should ensure free of cost replacement of any part / components in the equipments, that undergoes wear and tear during guaranty period.

  - list  of  customers to whom similar welding rectifier have been supplied &working

satisfactory.