| 1.0 | RAW MATERIALS & BOUGHT OUT ITEMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Steels, Structures |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | MTC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.2 | Pipes |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat |  |  | MTC/TC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.21 |  |  | Hydro Test | Major | Hydro Test | 100% |  |  |  | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.3 | Pipe Fittings |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat |  |  | MTC, NDE Report | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.31 |  |  | NDE # | Major | MPI/LPT | 20% |  |  |  | √ | 2 |  | 1 | # Refer Note:1 |  |  |  |  |  |
| 1.4 | Flanges |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat |  |  | MTC/ TC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.41 |  |  | Rating, Type etc. | Major | Visual | 100% |  |  |  | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.5 | Plates |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat |  |  | MTC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.51 |  |  | UT if thickness >=16mm | Major | Chem., Mech. Analysis | 100% |  |  | IR | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.6 | Valves |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat |  |  | MTC/TC, NDE Report, Type Test Certificate | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.61 |  |  | Hydro Test, NDE, Fire Safe certificate, etc. # | Major | Hydro, NDE | As per BHEL Spec/ Datasheet |  |  |  | √ | 2 |  | 1 | # as applicable |  |  |  |  |  |
| 1.7 | Control Valves |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | MTC/TC, NDE Report, Type Test Certificate | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.71 |  |  | Seat Leakage test, Hydro test, Cv test, Actuator pneumatic test, Functional, Completeness# | Major | TC Review | 100% |  |  | MTC/TC, NDE Report, Type Test Certificate | √ | 2 |  | 1 | # as applicable |  |  |  |  |  |
| 1.8 | Safety Relief Valve |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat |  |  | MTC/TC, | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.81 |  |  | Hydro Test, Set Pressure Test | Major | Hydro, Functional | 100% |  |  |  | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.9 | Instruments |  | Chemical & Mechanical Properties, Calibration | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat, 100% |  |  | MTC/TC, Type Test Certificate | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.91 |  |  | Hydro test(as applicable), Type Test(Weather Proof & Hazardous area certification etc.)# | Major | Hydro, Type Test | 100% |  |  |  | √ | 2 |  | 1 | # as applicable |  |  |  |  |  |
| 1.10 | Junction Box |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat |  |  | MTC/TC, Type Test Certificate | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.10.1 |  |  | Type Tests (Weather Proof & Hazardous area certification etc.) | Major | Type Test | 100% |  |  |  | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.11 | Strainer |  | Chemical & Mechanical Properties ,Rating, Hydro Test | Major | Chem., Mech. Analysis, Hydro test | One per Lot/Batch /Heat, 100% | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | MTC/TC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.12 | Filter Elements |  | Chemical & Mechanical Properties, Degree of filtration | Major | Chem., Mech. Analysis, Testing | One per Lot/Batch /Heat |  |  | MTC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.13 | Cable & Cable Gland |  | Chemical & Mechanical Properties, Routine test, Type test | Major | Chem., Mech. Analysis | One per Lot/Batch /Heat |  |  | MTC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.14 | Local Gauge Board |  | Chemical Analysis, Routine tests, Weather proof protection | Major | Chem. Analysis, Routine test, IP Cert. | 100% |  |  | MTC, TC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.15 | Heater Element, Baffle Plate, Terminal box |  | Chemical & Mechanical Properties, Type test (IP, Ex-proof) | Major | Visual | 100% |  |  | MTC, TC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.16 | Gaskets , Fasteners, Lugs |  | Chemical & Mechanical Properties | Major | Visual | 100% |  |  | MTC, TC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 1.17 | Instruments Tubing, Ferrule, Bite coupling and Fitting |  | Chemical & Mechanical Properties | Major | Chem., Mech. Analysis | 100% |  |  | MTC | √ | 2 |  | 1 |  |  |  |  |  |  |
| 2.0 | INPROCESS INSPECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | For SS & AS Material | PMI |  | Major | PMI Check | Random | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | TC | √ | 2 | 1 | - |  |  |  |  |  |  |
| 2.2 | Fabrication of vessels and piping | WPS, PQR, WQR |  | Major | Doc. Verification | 100% |  |  | WPS, PQR, WQR | - | 2 | - | 1 |  |  |  |  |  |  |
| 2.21 |  | Fit up of shell, shell to disc ends, shell to nozzle, nozzle to flange etc. |  | Major | Doc. Verification | 100% |  |  | Log book/register /IR | √ | 2 |  | 1 |  |  |  |  |  |  |
| 2.3 | NDE on Welds | Radiography |  | Major | RT | As per BHEL Spec |  |  | NDE Reports, IR | √ | 2 | - | 1 |  |  |  |  |  |  |
| 2.31 |  | Liquid penetrant |  | Major | LPT |  |  |  |  | √ | 2 | 1 | - |  |  |  |  |  |  |
| 2.4 | Vessels and Piping | Thickness check |  | Major | Thickness check | Random |  |  | TC/IR | √ | 2 | 1 | - |  |  |  |  |  |  |
| 2.41 | Hydro test of vessels and piping | Hydro Test |  | Major | Hydro Test | 100% |  |  | TC | √ | 2 | 1 | - |  |  |  |  |  |  |
| 2.5 | Skid Assembly | Visual, Dimension, Alignment and Orientation |  | Major | Visual, Measurement | 100% |  |  | IR | - | 2 | 2 | - |  |  |  |  |  |  |
| 3.0 | FINAL INSPECTION & TESTING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A | SCRUBBER AND FILTER SEPERATOR SKID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Skid Assembly | Visual, Dimensional, Alignment, BOM |  | Major | Visual, Measurement | 100% | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | IR | √ | 2 | 1,3 |  |  |  |  |  |  |  |
| 3.12 |  | Hydro test of assembled skid |  | Critical | Hydro Test | 100% |  |  | IR | √ | 2 | 1,3 |  |  |  |  |  |  |  |
| 3.13 |  | Pneumatic leak test of assembled skid |  | Major | Hydro Test | 100% |  |  | IR | √ | 2 | 1,3 |  |  |  |  |  |  |  |
| B | ELECTRIC HEATER SKID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.2 | Skid Assembly | Visual, Dimensional, Alignment, BOM |  | Major | Visual, Measurement | 100% | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.21 |  | Hydro test of assembled skid |  | Critical | Hydro Test | 100% |  |  | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.22 |  | Pneumatic leak test of assembled skid |  | Major | Pneumatic Test | 100% |  |  | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.23 |  | HV/IR , Resistance, Wattage test |  | Major | Testing | 100% |  |  | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.3 | Control Panel /System | Demonstration of proper functioning of all hardware, Alarm simulations, Operator interface functions etc. |  | Major | Functional | 100% |  |  | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| C | STEAM HEATER SKID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.4 | Skid Assembly | Visual, Dimensional, Alignment, BOM |  | Major | Visual, Measurement | 100% | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.41 |  | Hydro test of assembled skid |  | Critical | Hydro Test | 100% |  |  | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.42 |  | Pneumatic leak test of assembled skid |  | Major | Hydro Test | 100% |  |  | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| D | FINE FILTER SKID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.5 | Skid Assembly | Visual, Dimensional, Alignment, BOM |  | Major | Visual, Measurement | 100% | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.51 |  | Hydro test of assembled skid |  | Critical | Hydro Test | 100% |  |  | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.52 |  | Pneumatic leak test of assembled skid |  | Major | Hydro Test | 100% |  |  | IR | √ | 2 | 1 |  |  |  |  |  |  |  |
| 3.6 | Commissioning / Mandatory Spares | Visual, TC verification |  | Major | Visual, Doc. Verification | 100% |  |  | IR | √ | 2 | 1 |  | 10% witness by BHEL |  |  |  |  |  |
| 4.0 | PRESERVATION & PACKING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 |  | Surface preparation, Painting (Shade, DFT)# Marking |  | Major | Visual, Measurement | Random | BHEL Spec/ Appd. Dwgs/ Datasheet | BHEL Spec/ Appd. Dwgs/ Datasheet | TC | √ | 2 | 1 |  | #as applicable |  |  |  |  |  |
| 4.2 |  | Packing# |  | Major | Visual | Random |  |  | Packing List | √ | 2 |  | 1 | # Sea worthy packing in case of Export orders |  |  |  |  |  |

Notes:-

- MPI/LPT Should be carried on Knuckle of Reducers, Outer bend portion of Elbows and Neck portion of Tees.

- CS & AS (>=2”) Material – MPI         b) SS Material (>=2”) – LPT.

- This Standard MQP should be read along with specification (Latest revision as per PO), approved drawings & approved datasheet.

- Drawing/datasheet shall prevail over quality plan for contradiction if any.

- All type test reports submitted shall not be older than 5 yrs from the date of purchase order.

- Any project/customer specific requirements which shall be notified have to be fulfilled by the vendor at the time of execution of order.

- The applicability of above items/products depends on scope of supply. Same to be considered accordingly during inspections.

Abbreviations:-

MTC	-	Material Test certificate					WPS	-	Welding Procedure Specification

PQR	-	Procedure Qualification Record 				MPI	-	Magnetic Particle Inspection

WPQ	-	Welder Performance Qualification			LPT	-	Liquid Penetrant Test

NDE	-	Non Destructive Examination				TC	-	Test Certificate

PMI	-	Positive Material Identification				CS	-	Carbon Steel

SS	-	Stainless Steel						DFT	-	Dry Film Thickness

IR	-	Inspection Report