| Tender No: BHEL:MSTPP:AUX:PUR:20:03 |  |
| --- | --- |
| PREAMBLE OF BOQ |  |
| 1 | This preamble forms part of tender document and schedule of items. The tenderer should read this preamble carefully before filling in rates for various items. Clauses under this preamble shall be read in conjunction with various volumes of tender and other tender sections as applicable and shall have precedence over any contrary statement mentioned any where in this document. |
| 2 | The work shall be carried out strictly as per specifications, description of the items in these schedule and / or engineer’s instructions. Drawings enclosed with the tender are only preliminary and for tender purposes and giving some idea of the work involved. The work is to be executed as per drawings & documents, which shall be furnished during execution. |
| 3 | Items of work provided in this schedule but not covered in this specification shall be executed strictly as per instruction of the engineer. |
| 4 | Unless specifically mentioned otherwise in the tender document, the tenderer shall quote for the finished items and shall provide for the complete cost towards power, fuel, tools, tackles, equipment, constructional plants, temporary works, labour, dismantling of all temporary piping, structures, valves, pumps, tanks & other misc. equipment, strengthening of roads/culverts/bridges etc. including arranging all clearances etc. required for carrying out different activities & tests, materials, levies, taxes (except service tax), transport, layout, repairs, rectification, maintenance till handing over, supervisions, colonies, shops, establishments, overheads, profits and all incidental items not specifically mentioned but reasonably implied and necessary to complete the work according to the complete tender document and this schedule. |
| 5 | The quantities of the various items mentioned in this schedule of items are approximate, based on very preliminary information and may vary to any extent or be deleted altogether. The quoted rates of each item will remain firm throughout the period of execution including extension, for reasons whatsoever, as long as variation in the total value of work executed under any part of this contract including extra items, if any but excluding any price variation remains, within plus minus thirty percent (± 30%) of the awarded price. |
| 6 | Prior written approval of BHEL shall be sought by the contractor in case quantity variation of any item crosses +50% (plus fifty percent) limit during execution and approval to be obtained before execution of further quantity for this item. |
| 7 | In case Sealed /Paper Price Bids are opened for finalisation of the tender, for any Item Rate/BOQ based contract, possibility of variation of quantity/ addition/deletion of items can not be ruled out. Under such circumstances, after execution of work, if it is observed that standing as L-1 is changed based on actual quantity executed, the bidder shall give suitable rebate to maintain your standing as L-1. Since this aspect can be assessed at the end of execution, necessary adjustment will be effected at the end of execution in final bill.This condition shall not be applicable where the tender is finalised through Reverse Auction. |
| 8 | BHEL reserve the right to rationalize the rates, quoted by L-1 bidder against unit rate items and/ or other optional items with respect to item-wise lowest rates (amongst the participating bidders), before placement of order. |
| 9 | The rates quoted shall be inclusive of cleaning of site of any vegetation, dressing , clearing of old structures and leveling etc. including fixing of grid pillars, benchmarks etc. required for commencement of site activities. No separate payment will be made towards the same. However, if separate rate for such item is avalable in the rate schedule, the same shall be considered. |
| 10 | Rates shall be quoted in figures and in words in clear legible writing. No overwriting is allowed. All scoring and cancellations should be countersigned and in case of illegibility the interpretation of engineer shall be final. All entries shall be in English language. |
| 11 | All works item wise shall be measured upon completion and paid for at the rates quoted and accepted as per BHEL approved payment schedule/billing break-up. |
| 12 | The tender shall be deemed to have visited site and made himself aware of all the site conditions, studied the specifications and details of work to the done within the time schedule attached and to have acquainted himself of the conditions prevailing at site. |
| 13 | No splitting of the job is envisaged. Decision of BHEL in this regard shall be final and binding to the bidders. |
| 14 | Bidders are not allowed to alter the Price Schedule format including item description,quantity etc. and the offer is liable for rejection if the bidders submit their prices in Price Schedules modified by them.BHEL reserves the right to reject the offers of bidders who submit offers in Price Formats which are modiified/altered by them. Also putting any comments instead of rates/price in the designated column of the rate schedule shall make the offer liable for rejection. |
| 15 | Bidders to note that for Civil packages, against a particular item against a ST No. appearing in more than one schedule of the BOQ, same rate must be quoted in all schedules for that particular items with same descriptions. If by error, different rates are quoted in different schedules for same ST No.(i.e. item with same description), then the higher of the rates shall be considered for evaluation but awarding shall be done with the lower rate, if the bidder becomes L-1.” |
| 16 | For Lumpsum Contract : The items/components indicated in the tender is indicative and may vary to any extent. No compensation shall be payable in case of any variation in the items/components listed in the bill of quantities if the executed weight remains within the variation limit. However, in case of deletion of any item or addition of new items over and above the items listed or variation of existing quantity beyond variation limit specified, adjustment (i.e. Payment or recovery as the case may be ) shall be done on pro-rata basis based on the Rate per MT worked out from the quoted lump-sum Price and the total weight of components listed /indicated in price schedule plus 15% weight variation limit. |
| 17 | Engineer’s decision shall be final and binding on the contractor regarding clarification of items in the schedule with respect to the other sections/volumes of the contract. |
| 18 | Size and weights of various items are mentioned in the attached BOQ cum rate/price schedule for reference purpose only & these shall not be taken into consideration for quoting/calculating amount in the rate schedule. These shall be utilised as per relevant sections of tender. Bidders shall quote for each item in the rate column, taking unit as mentioned in the quantity column. Rates shall be filled in both figures and words. Amount shall be calculated based upon these rates multiplied by the mentioned quantity for the respective items. |
| 19 | Bidder's Total price shall be considered for evaluation unless stated otherwise. |
| 20 | In case of BOP packages, if Bidder does not quote/indicate the price for freight chagses against indicated rate schedule, the same shall be considered as 2% of basic price and adjusted with the total quoated price against each item keeping the total quoted price unaltered. |

PRICE BID

JOB: PACKAGE 10U : PROVIDING AND FIXING OF ACID / ALKALI RESISTANT ( CHEMICAL RESISTANT ) TILES WITH POTTASIUM SILICATE BEDDING MORTAR ETC COMPLETE CONFORMING TO RELEVANT BS OR EQUIVALENT INTERNATIONAL CODE FOR AUX & BOP BUILDINGS FOR 2X660 MW MAITREE , RAMPAL PROJECT , BANGLADESH

S. N.; Description of item; Unit; Quantity; All inclusive F.O.R. destination Unit price excluding Indian GST or Bangladesh VAT (in USD) (in figure); All inclusive total F.O.R. destination price excluding Indian GST or Bangladesh VAT (in USD) (In figure); All inclusive total F.O.R. destination price excluding Indian GST or Bangladesh VAT (in USD) in words. 1; 2; 3; 4; 5; 6=5x4; 7. 1400 FLOORING AND SKIRTING. Flooring and skirting at any level including base layer, labour, material (unless otherwise specified in BOQ/specification), equipments, transportation, handling, curing, polishing etc. at any level as per relevant laws of Bangladesh, relavant BS Standards or Equivalent International standards approved by BIFPCL/BHEL, specification, drawings and as directed by engineer - in - charge.. 1422; Providing & fixing Acid / Alkali resistant (Chemical resistant) tiles confirming to relevant BS or equivalent international code in flooring/Dado and shall be laid over bitumastic lining of min 12mm thick ( to be laid in layers of 6mm each). The tiles shall be applied with 6mm thick Potassium Silicate bedding mortar as per relevant BS or equivalent international code and including preparation of surface, application of bitumen primer, curing etc. all complete for following thicknesses. The tiles should be abrasion resistant & durable.. a; 20mm thick; Sqm; 3702. 1423; Providing & fixing Acid / Alkali resistant (Chemical resistant) tiles(tiles should be abrasion resistant & durable) in flooring/Dado and shall be laid over bitumastic lining of min 12mm thick ( to be laid in layers of 6mm each). The tiles shall be applied with 6mm thick Potassium Silicate bedding mortar and including preparation of surface, application of bitumen primer, curing, pointing joint of bedding 20mm deepx6mm wide with epoxy/Furane morteretc. all complete as per manufacturers guidelines, specifications, drawing, relevant BS or equivalent international standards and and/or as approved by BIFPCL/BHEL for following minimum thicknesses.. a; 20mm thick; Sqm; 1065. b; 38mm thick; Sqm; 250. 1424; Providing & fixing chemical resistant (AR)(Acid / Alkali) bricks (75mm thick) conforming to relevant BS or equivalent international code in the floor of neutralization pit. Surface on which lining to be applied shall be prepared in accordance with relevant BS or equivalent international code. Bitumen primer as per relevant BS or equivalent international code followed by 18mm thick bituminastic followed by 6mm thick potassium silicate mortar bedding shall be provided before laying AR bricks. The joints between AR bricks shall be filled with resin type of mortar conforming to relevant BS or equivalent international code, seal coat of readymade epoxy paint shall be provided on joints to cover up any porosity that may be left in mortar. End sealing shall be done with bituminastic AR bricks shall be laid with 6mm wide & 50mm deep pointing (epoxy / furnace / CNSL) & acid curing shall be done all complete as per specification.; sqm; 300. A1424; Providing & fixing chemical resistant (AR)(Acid / Alkali) tiles (75mm thick) in the floor of neutralization pit. Surface on which lining to be applied shall be prepared in accordance with relevant BS or equivalent international code. Bitumen primer followed by 18mm thick Bitumenastic followed by 6mm thick potassium silicate mortar bedding shall be provided before laying AR bricks. The joints between AR tiles shall be filled with resin type of mortar, seal coat of readymade epoxy paint shall be provided on joints to cover up any porosity that may be left in mortar. End sealing shall be done with Bitumenastic AR bricks shall be laid with 6mm wide & 50mm deep pointing (epoxy / furnace / CNSL) & acid curing shall be done all complete as per manufacturers guidelines, specifications, drawing, relevant BS or equivalent international standards and and/or as approved by BIFPCL/BHEL.; sqm; 400. B1424; Providing & fixing chemical resistant (AR)(Acid / Alkali) bricks (115 mm thick) in the walls of neutralization pit. Surface on which lining to be applied shall be prepared in accordance with relevant BS or equivalent international standards. Bitumen primer followed by 18mm thick Bitumenastic followed by 6mm thick potassium silicate mortar bedding shall be provided before laying AR bricks. The joints between AR bricks shall be filled with resin type of mortar, seal coat of readymade epoxy paint shall be provided on joints to cover up any porosity that may be left in mortar. End sealing shall be done with Bitumenastic AR bricks shall be laid with 6mm wide & 50mm deep pointing (epoxy / furnace / CNSL) & acid curing shall be done all complete as per manufacturers guidelines, specifications, drawing, relevant BS or equivalent international standards and and/or as approved by BIFPCL/BHEL.; sqm; 636. GRAND TOTAL. NOTE: - EVALUATION SHALL BE ON THE GRAND TOTAL AMOUNT EXCLUDING VAT (IN USD.)”. - BIDDER’S QUOTED "ALL INCLUSIVE F.O.R. DESTINATION GRAND TOTAL PRICE ” SHALL INCLUDE ALL STATUTORY DUTIES, TAXES AND ALL OTHER CHARGES EXCLUDING VAT ON F.O.R. DESTINATION BASIS AT BHEL INCLUDING SUPPLY AND INSTALLATION , PSER’s MAITREE STPP BANGLADESH SITE” ON DOOR DELIVERY BASIS AS PER DEFINITION IN GENERAL TERMS & CONDITIONS OF TENDER DOCUMENT. - BIDDER SHALL UNDERTAKE TRANSIT INSURANCE, FOR MATERIAL TRANSPORTED AND MADE AVAILABLE AT BHEL'S SITE ON F.O.R. SITE BASIS. - BIDDER’S QUOTED RATE/PRICE SHALL REMAIN FIRM FOR OFFER VALIDITY PERIOD , SUPPLIES & INSTALLATION. - WHILE QUOTING, BIDDER SHALL TAKE INTO CONSIDERATION ALL OTHER TERMS AND CONDITIONS SPECIFIED IN THE TENDER DOCUMENT INCLUDING GENERAL CONDITIONS OF CONTRACT (PART - B) AND SPECIAL CONDITIONS OF CONTRACT (PART - C). - ANY OTHER DOCUMENT SUBMITTED BY BY BIDDER) IN PRICE BID/SCHEDULE APART FROM TENDERED PRICE BID/SCHEDULE SHALL NOT BE TAKEN INTO COGNIZANCE FOR EVALUATION OF OFFER. ANY MODIFICATION IN PRICE BID/SCHEDULE IN ANY WAY BY BIDDER IS NOT ACCEPTABLE & SHALL BE CONSIDERED AS TAMPERING OF TENDER DOCUMENT. - For Bangladeshi Bidders : Bidder to take into consideration the provision of Duty Drawback / Export Incentive in Bangladesh, as applicable. As such, while offering the rates, the supplier may take into account the benefit of above provisions, as the cost of input to the supplier will be net of such incentives and adjust their offer price accordingly to make it more competitive. However, effect of any change in the Duty Drawback / Export Incentive Rules shall be mutually decided by bidder & BHEL on the basis of of relevant documents. - Bidder’s quoted rate/price shall remain firm for offer validity period & supplies. - BHEL reserve the right to rationalize the rates, quoted by L-1 bidder against unit rate/ optional items with respect to item-wise lowest rates (amongst participating bidders), before placement of order. - Any other document submitted by by bidder) in price bid/schedule apart from tendered price bid/schedule shall not be taken into cognizance for evaluation of offer. Any modification in price bid/schedule in any way by bidder is not acceptable & shall be considered as tampering of tender document. - Bidder shall be provided RFC drawings for supply and installation. Bidder has to prepare BOQ accordingly. Payment shall be made for complete finished item only as per the RFC drawing. No wastage is payable.

- SIGNATURE OF THE BIDDER WITH SEAL & DATE